WARNING ：

Read this

Manual ,and any

Supplements

Carefully

Before

Operating

Vehicle

·Assembly

·Operation

·Maintenance

THIS VEHICLE’S

NOT DESIGNED

FORUSEON

RENTALTRACKS

OR RACING

OWNER’S/OPERATOR’S

MANUAL

BEFORE OPERATING THIS VEHICLE,THE OWNER AND EACH OPERATOR MUST UNDERSTAND THAT THIS VEHICLE WAS NOT DESIGNED OR MANUFACTURED TO MEET DEMANDS FOR USE ON PUBLIC ROADS,STREETS ,HIGHWAYS.AND THOROUGHFARES AND YOU HAVE TO UNDERSTAND ALL THE INSTRUCTIONS FOR PROPER ASSEMBLY AND SAFE OPERATION AS  WELL  AS THE ISTRUCTIONS CONCERNING THE ENGINE AND ALL OTHER PORTIONS OF THE VEHICLE .

NOTE:

A storage area/toolbox has been provided under the seat of the ATV for storage of this manual, any addendums and the provided tire pressure gauge. Please keep this manual and any addendums in the storage area/toolbox so that it can be found easily and referenced when needed.

# CONTENTS

- INTRODUCTION…………………………………………………………………………4

- UNDERSTANDING WARNING…………………………………………………………6

- SAFETY WARNING………………………………………………………………………7

- DAILY PRE-RIDE INSPECTION ………………………………………………………11

- OPERATION WARNING…………………………………………………………………11

- V.I.N.……………………………………………………………………………………… 20

- CONTROL AND PARTS FUNCTIONS ……………………………………………… 21

- STARTING THE ENGINE ………………………………………………………………27

- VEHICLE BREAK-IN PERIOD………………………………………………………… 28

- RIDING WEAR……………………………………………………………………………29

- CARRYING LOADS………………………………………………………………………29

- RIDING ……………………………………………………………………………………30

- CVT SYSTEM ……………………………………………………………………………36

- BATTERY …………………………………………………………………………………37

- EXHAUST SYSTEM………………………………………………………………………38

- MAINTENACE ……………………………………………………………………………39

- TROUBLE SHOOTING……………………………………………………………………51

- TOOLS ……………………………………………………………………………………54

- SPECIFICATION …………………………………………………………………………55

20. WIRING DIAGRAM ……………………………………………………………………………… 57

- INTRODUCTION

Congratulation on the purchase of this ALL Terrain Vehicle(ATV). We take pride in offering you this product engineered and manufactured to the highest performance and quality standards. We are sure that you will enjoy superior levels of performance, reliability, riding comfort, and safety.

This manual is provided to help the owner and operators of the ATV become familiar with the operations, characteristics, and other features of the ATV. The manual also covers information on the care and maintenance of the ATV.

Please read this manual carefully. The information contained in this owner’s Manual, the warning labels on this vehicle will help you to understand the safe use and maintenance of your ATV.Pls make sure that you understand and follow all warning and instructions in this manual

# Important Safety Notice

Never make any modifications to the engine, drive system, mechanical or electrical system of your ATV. Never install aftermarket parts or accessories intended to increase the speed or power of your ATV.

Failure to follow these warning increases the possibility or accidents leading to DEATH or SERIOUS INJURY!

# NOTE

Addition and use of certain accessories including, (but not limited to ) mowers, blades, sprayers, winches and windshields will change the handing characteristics of your ATV.

Please be certain that any accessory added has been approved by your dealer.

## Practice Responsible ATV Riding

Make sure that you understand and follow all local, state/province, and federal riding laws and requirements. Many parks, Recreation Areas, and Trails that allow ATV riding have rules beyond those listed above. Be smart, check before you ride.

Remember……Respect your vehicle, respect the environment and respect the property of others. You are responsible for your safety and the safety of others around you when you ride!

An ATV CAN BE HAZARDOUS TO OPERATE: An ATV handles differently from other vehicles including motorcycles and cars. A collision rollover can occur quickly, even during routine maneuvers such as turning and driving on hills or over obstacles, if you fail to take proper precautions.

SERIOUS INJURY OR DEATH can result from your not following these instructions：

- Read this manual and all labels carefully and follow the operating procedures described.

- Never operate an ATV without proper instruction. Take a training course. Beginners should receive training from a certified instructor. Contact an authorized ATV dealer to find out about the training courses nearest you.

- Never allow anyone who is not an adult to operate this ATV.

- Never permit a guest to operate this ATV unless the guest has read this manual and all products labels, and has completed a cetified training course.

- Never operate an ATV on public street,road or highway,even a dirt or gravel area.

- Never operate an ATV without wearing an approved helmet that fits properly. You should also wear eye protection (goggles or face shield), gloves, boots, long-sleeved shirt or jacket, and long pants.

- Never consume alcohol or drugs before or while operating this ATV.

- Never operate at excessive speeds. Always travel at speed which is proper for the terrain, visibility and operating conditions, and your experience.

- Never attempt wheelies, jumps or other stunts.

- Always inspect your ATV each time you use it to make sure it is in safe operating condition. Always follow the inspection and maintenance procedures and schedules described in this manual.

- Always keep both hands on the handlebars and both feet on the footrests of the ATV during operation

- Always go slowly and be extra careful when operating on unfamiliar terrain. Always be alert to changing terrain conditions when operating the ATV.

- Never operating on excessively rough, slippery or loose terrain

- Always follow proper procedures for turning as described in this manual. Practice turning at low speeds before attempting to turn at faster speeds. Do not turn at excessive speed.

- Always have the ATV checked by an authorized dealer if it has been involved in an accident.

- Never operate ATV on hills too steep for the ATV to climb or beyond your abilities. Practice on smaller hills before attempting larger hills.

- Always follow proper procedures for climbing hills as described in this manual. Check the terrain carefully before you start up any hill. Never climbing hills with excessively slippery or loose surfaces. Shift your weight forward. Never open throttle suddenly or make sudden gear changes. Never go over the top of any hill at high speed.

- Always follow proper procedures for going down hills and for braking on hills as described in this manual. Check the terrain carefully before you start down any hill. Shift your weight backward. Never go down a hill at high speed. Avoid going down a hill at an angle that would cause the vehicle to lean sharply to one side. Go straight down the hill where possible.

- Always follow proper procedures for crossing the side of a hill as described in this manual. Avoid hills with excessively slippery or loose surfaces. Shift your weight to the uphill side of the ATV. Never attempt to turn the ATV around on any hill until you have mastered the turning technique described in this manual on level ground. Avoid crossing the side of a steep hill if possible.

- Always use proper procedures if you stall or roll backwards when climbing a hill. To avoid stalling, maintain a steady speed when climbing a hill. If you stall or roll backwards, follow the special procedure for braking described in this manual. Dismount on the uphill side or to either side if pointed straight uphill, turn the ATV around and remount, following the procedure described in this manual.

- Always check for obstacles before operating in a new area. Never attempt to operate over large obstacles, such as large rocks or fallen trees. Always follow proper procedures when operating over obstacles as described in this manual.

- Always be careful of skidding of sliding. On slippery surfaces, such as ice, go slowly and be very cautious in order to reduce the chance of skidding or sliding out of control.

- Avoid operating the ATV through deep or fast flowing water, avoid water which exceeds the recommended maximum depth , go slowly, balance your weight carefully avoiding sudden movements , maintain a slow and steady forward motion, do not make sudden turns or stops and do not make sudden throttle changes.

- Wet brakes may have reduces stopping ability. Test your brakes after leaving water. If necessary apply them lightly several times to let friction dry out the pads.

- Always be sure there are no obstacles or people behind you when you operate in reverse. When it is safe to proceed in reverse, go slowly. Avoid turning at sharp angles in reverse.

- Always use the size and type tires specified in this manual. Always maintain proper tire pressure as described in this manual.

- Never modify an ATV through improper installation or use of accessories.

- Never exceed the stated load capacity for an ATV. Cargo should be properly distributed and secured load capacity for an ATV. Cargo should be properly distributed and securely attached. Reduce speed and follow instructions in this manual for carrying cargo or pulling a trailer. Allow greater distance for braking.

ATTENTION:

This is an ADULT VEHICLE ONLY; not toy. Operation is prohibited for anyone under 16 years of age. READ AND UNDERSTAND WARNINGS AND OWNER’S MANUAL BEFORE OPERATION.

# KNOW YOUR VEHICLE BEFORE YOU BEGIN RIDING!

Read this manual thoroughly referring to the various areas which are being discussed on your machine. Operating this vehicle carries with the responsibilities for your personal safety,the safety of others,and the protection of our environment.

# SAFETY TRAINING

We strongly recommend that all new ATV riders attend a professional training class. Once you have completed this training,you will be reimbursed for the cost of the class for all first time riders in your immediate family. Fill out and return the Rider Training Coupon that you received with your new ATV to receive the reimbursement plus a certificate for up to $100 in free merchandise.

NOTE:Illustrations used in this manual are for general representation only. Your model may differ.

- UNDERSTANDING WARNING

SAFETY ALERT

WARNINGS Identify special instructions or procedures that, if not correctly followed, could result in personal injury, or loss of life. Read all WARNINGS in this manual carefully. Follow their instructions to remain safe. The following precautionary signal words are used throughout this manual to convey the following messages:

# This is the safety alert symbol. When you see this symbol on your machine or in this manual, be alert to the potential for personal

# Injury. Your safety is involved!

### WARNING

Indicates a potential hazard that could result in severe injury or death.

CAUTION

Indicates a potential hazard which may result in minor personal injury or damage to the ATV.

#### CAUTION

Indicates a situation that can result in damage to the machine.

NOTE  The word “NOTE:” in this manual will alert you to key information or instruction

- SAFETY WARNING

NOTE:

Warning decals have placed on the vehicle for your protection. Read and follow the instructions on each decal carefully. In the event any decal becomes illegible or comes off, contact your dealer for a replacement.

WARNING

Operation of this ATV by children under the age of 16 increases the risk of severe injury or death. NEVER permit children under age 16 to operate this ATV

1.

WARNING

ENGINE EXHUAUST FROM THIS PRODUCT CONTAINS CHEMICALS KNOWN, IN CERTAIN QUATITIES, TO CAUSE CANCER, BIRTH DEFECTS, OR OTHER REPRODUCTIVE HARM.

2.

WARNING

Overloading of the carrying rack can cause loss of stability or control of the ATV.

Do not tow from the rack or bumper. Loss of stability or tip over may result.

Do not exceed the load capacity of the rack.

Load capacity of this front carrying rack is 40 Ibs/18kg

3.

# Improper ATV use can result in SEVERE INJURY or DEATH

ALWAYS USE AN           NEVER USE             NEVER USE DRUGS

APPROVED HELMET       ON PUBIC ROADS         OR ALCOHOL

NEVER operate:

- Without proper training or instruction.

- At speeds too fast beyond your skills or the conditions.

- On public roads collision can occur with another vehicle.

ALWAYS:

- Use proper riding techniques to avoid vehicle overturns on hills and rough terrain and in turns

- Avoid paved surfaces-pavement may seriously affect handling and control

LOCATE AND REAR OWNER’S MANUAL

FOLLOW ALL INSTRUCTIONS AND WARNINGS

4.

WARNING

##### Hot surfaces

Never touch engine or exhaust system after running engine, until they have cooled.

WARNING

THIS GUARD MUST BE IN PLACE DURING OPERATION OF THE ENGINE. KEEP HANDS, FEET, HAIR AND LOOSE GARMENTS ALWAYS FROM THE ENGINE, CHAIN AND DRIVE CONPONENTS.

DRIVE CONPONENTS.

5                                   6

WARNING

Overloading of the carrying rack can cause loss of stability or control of the ATV. Towing from the rack can cause tip over of the ATV. Do not exceed the load capacity of the rack. Do not tow from this rack. Load capacity of this rear carrying      rack is 80Ibs/36kg.

7

WARNING

- Shoulder length and longer hair or loose clothing can get caught in moving parts behind or below the seat.

- Restrain hair at or above shoulder level. Restrained hair must not interfere with the proper fit of the helmet.

- Securely restrain anything loose that reaches behind of below the seat.

Guards are supplementary Safety Devices.

The restraint of hair and clothing is the primary means of avoiding entanglement death or injury.

8

WARNING

Improper tire pressure or overloading can

cause loss of control.

Loss of control can result in severe injury or death.

ALWAYS maintain proper tire pressure as indicated on the sidewall of your ATV tires.

NEVER exceed the vehicle load capacity of 350 lbs/150kg.

Combined rider and payload.

9

## WARNING

Failure to understand and follow warning and instructions for the safe use and maintenance of this product may result in death or injury!

This information is contained in the warning labels. Owner’s Manual &Supplements. Make sure that you understand and follow all warning and instructions in this material.

10

## WARNING

Never operate this vehicle on HILLS steeper than 15%.

To prevent flip over on hilly terrain, use the throttle and brake gradually.

Operation of the ATV in reverse, ever at low speeds can be dangerous. To prevent flipping over, avoid sudden braking and sharp turns

11

12

WARNING

Do not remove the radiator cap when the engine and radiator are hot. Scalding hot fluid and steam may be blown out under pressure, which could cause serious injury. When the engine has cooled, open the radiator cap as follows:

Place a thick rag or a towel over the radiator cap. Slowly rotate the cap counterclockwise toward the detent. This allows any residual pressure to escape. When any hissing sound has stopped. Press down on the cap while turning counter clockwise and remove it. (This label is pasted on the frame under the front cover.)

WARNING

4.DAILY PRE-RIDE INSPECTION

You must inspect your ATV each time before riding to engine it is in proper working order, if proper inspection is not done, severe injury or death could result.

Use the following checklist to verity your machine is in proper working order each time you ride.

##### Item/inspection procedure

- Tires-check condition and pressures.

- Fuel tank-fill the tank to its proper lever.

- All brakes-check operation, adjustment and fluid lever

- Throttle-check for free operation and closing.

- Headlight/taillight/brake light- checks operation of all indicator lights and switches.

- Engine stop switch-check for proper function.

- Wheels-check for tightness of wheel nuts and axle nuts:

- Air cleaner element-check for dirt: clean or replace.

- Steering-check for free operation not any unusual looseness in any area.

- Loose parts-visually inspect vehicle for any damaged components or loose nuts/bolts or fasteners.

- Operators helmets, goggles and clothing.

- Engine coolant check for proper lever at the recovery bottle.

- OPERATION WARNINGS

WARNING

##### POTENTIAL HAZARD

Operating this ATV without proper instruction.

##### WHAT CAN HAPPEN

The risk of an accident is greatly increased if operator

Does not know how to operate the ATV properly in different situations and on different types of terrain.

##### HOW TO AVIOD THE HAZARD

Beginning and inexperienced operation should complete the certified training course.

They should then regularly practice the skills learned in the course and the operating techniques described in the Owner’s Manual.

## WARNING

POTENTIAL HAZARD

Operating this ATV without wearing an approved helmet, eye protection and clothing.

##### WHAT CAN HAPPEN

Operating without an approved helmet increases your changes of a severe head injury or death in the event of an accident.

Operating without eye protection can result in an accident and increase your changes of a severe injury in the event of an accident.

##### HOW TO AVOID THE HAZARD

Always wear an approved helmet, which fits properly.

You should also wear: eye protection (goggles or face shield); gloves; boots long-sleeved shirt or jacket; and long pants.

# WARNING

POTENTIAL HAZARD

Operation this ATV on paved surfaces, including sidewalks, paths, parking lots, and driveways.

##### WHAT CAN HAPPEN

The ATV tires are designed for off-road use. Paved surfaces cause the vehicle to go out of control.

##### HOW TO AVOID THE HAZAED

Avoid operating the ATV on pavement.

## WARNING

POTENTIAL HAZARD

Operating this ATV on public streets, roads or highways.

WHAT CAN HAPPEN

You can collide with another vehicle

HOW TO AVOID THE HAZARD

Never operate this ATV on any public street, road or highway, even a dirt or gravel one.

## WARNING

POTENTIAL HAZARD

Operating this ATV after consuming alcohol or drugs.

WHAT CAN HAPPEN

Could seriously affect your judgment.

Could cause you to react more slowly.

Could affect your balance and perception.

Could result in an accident.

HOW TO AVOID THE HAZARD

Never consume alcohol or drugs before or while driving this ATV.

## WARNING

POTENTIAL HAZARD

Operating this ATV at excessive speeds.

##### WHAT CAN HAPPEN

Increases your chances of losing of the ATV,which can result in an accident.

##### HOW TO AVOID THE HAZARD

Always travel at a speed which is proper for the terrain,

Visbility and operating conditions; and your experience.

## WARNING

POTENTIAL HAZARD

Attempting wheelies, jumps and other stunts.

WHAT CAN HAPPEN

Increase the change of an accident, including an overturn.

HOW TO AVOID THE HAZARD

Never attempt stunts, such as wheelies or jumps.

## WARNING

POTENTAL HAZARD

Failure to inspect the ATV before operating.

Failure to properly maintain the ATV.

WHAT CAN HAPPEN

Increases the possibility of an accident or equipment damage.

HOW TO AVOID THE HAZARD

Always inspect your ATV each time you use it to make sure

The ATV is in safe operating condition.

Always follow the inspection and maintenance procedures and schedules described in he Owner’s Manual.

## WARNING

POTENTIAL HAZARD

Removing hands from handlebars or feet from footrests during operation

WHAT CAN HAPPEN

Removing even one hand or foot can reduce your ability to control the ATV or could cause you to lose your balance and fall off the ATV. If you remove a foot from the footrest, your foot or leg may come into contact with the rear wheels, which could injury you or cause an accident.

HOW TO AVOID THE HAZARD

Always keep both hands on the handlebars and both feet on the footrests of your ATV during operation.

## WARNING

POTENTIAL HAZARD

Failure to use extra care when operating this ATV on unfamiliar terrain.

WHAT CAN HAPPEN

You can come upon hidden rocks, bumps, or holes, without enough time to react.

Could result in the ATV overturning or going out of control.

HOW TO AVOID THE HAZARD

Go slowly and be extra careful when operating on unfamiliar terrain.

Always be alert to changing terrain conditions when operating the ATV.

## WARNING

POTENTIAL HAZRD

Failure to use extra care when operating on excessively rough,slippery or loose terrain.

##### WHAT CAN HAPPEN

Could cause loss of traction or vehicle control,which could result in an accident,including an overturn.

HOW TO AVOID THE HAZARD

Do not operate on excessively rough,slippery or loose terrain until you have learned and practiced the skills necessary to control the ATV on such terrain

Always be especially cautious on these kinds of terrain.

## WARNING

POTENTIAL HAZARD

Climbing hills improperly.

WHAT CAN HAPPEN

Could cause loss of control or cause ATV to overturn.

HOW TO AVOID THE HAZARD

Always follow proper procedures for climbing hills as described in the Owner’s Manual.

Always check the terrain carefully before you start up any hill.

Never climb hills excessively slippery or loose surfaces.

Shift your weight forward.

Never open the throttle suddenly. The ATV could flip over backwards

Never go over the top of any hill at high speed. An obstacle. A sharp drop or another vehicle or person could be on the other side of the hill.

## WARNING

POTENTIAL HAZARD

Turing improperly.

WHAT CAN HAPPEN

ATV could go out of control, causing a collision or overturn.

HOW TO AVOID THE HAZARD

Always follow proper procedures for turning as described in the owner’s Manual.

Practice turning at low speeds before attempting to turn at faster speeds.

Do not turn at excessive speed.

## WARNING

##### POTENTIAL HAZARD

Operating on excessively steep hills.

##### WHAT CAN HAPPEN

The vehicle can overtune more easily on extremely steep hills than on level surfaces or small hills

HOW TO AVOID THE HAZARD

Never operate the ATV on hills too steep for the ATV or for your abilities.

Practice on smaller hills before attempting large hills

Never operate ATV on hills steeper than 15%.

## WARNING

POTENTIAL HAZARD

Going down a hills improperly.

WHAT CAN HAPPEN

Could cause loss of control or cause ATV to overturn.

HOW TO AVOID THE HAZARD

Always follow proper procedures for going down hills as described in the owner’s Manual. NOTE:A special technique is required when braking as you go downhill.

Always check the terrain carefully before you start down any hill.

Shift your weight back ward.

Never go down a hill at high speed.

Avoid going down a hill at an angle which would cause the vehicle to lean sharply to one side. Go straight down the hill where possible.

## WARNING

POTENTIAL HAZARD

Improperly crossing hills or turning on hills.

WHAT CAN HAPPEN

Could cause loss of control or cause ATV to overturn.

HOW TO AVOID THE HAZARD

Never attempt to turn the ATV around on any hill until you have mastered the turning technique as described in the Owner’s Manual on level ground. Be very careful then turning on any hill.

Avoid crossing the side of a steep hill if possible.

When crossing the side of a hill:

Always follow proper procedures as described in the Owner’s Manual.

Avoid hills with excessively slippery or loose surfaces.

Shift your weight to the uphill side of the ATV

## WARNING

POTENTIAL HAZARD

Stalling, rolling backwards or improperly dismounting while climbing a hill

WHAT CAN HAPPEN

Could result in ATV overturning.

HOW TO AVOID THE HAZARD

Maintain steady speed when climbing a hill

If you lose all forward speed:

Keep weight uphill.

Apply the brakes.

Lock parking brake after you are stopped

If you begin rolling backwards;

Keep weight uphill; never apply engine power.

Never apply the rear brake while rolling back wards.

Apply the single-lever brake gradually

When fully stopped, apply rear drake as well, and then lock parking brake.

Dismount on uphill side, or to either side if pointed straight uphill.

Turn the ATV around and remount, following the procedure described in the owner’s Manual.

## WARNING

##### POTENTIAL HAZARD

Imporoperly operating over obstacles.

##### WHAT CAN HAPPEN

Could cause loss of control or a collision. Could cause the ATV to overturn.

##### HOW TO AVOID THE HAZARD

Before operating in a new area, check for obstacles.

Use extreme caution when riding over large obstacles.

Such as large rocks or fallen trees.

If you cannot avoid obstacles, always follow proper procedures as describes in the owner’s manual.

## WARNING

##### POTENTIAL HAZARD

Skidding or sliding.

##### WHAT CAN HAPPEN

You may lose control of the ATV.

You may also regain traction unexpectedly, which may cause

The ATV to overturn.

##### HOW TO AVOID THE HAZARD

On slippery surfaces, such as ice, go slowly and be very cautious in order to reduce the chance of skidding or sliding out of control.

## WARNING

##### POTENTIAL HAZARD

Operation this ATV through deep or fast flowing water.

##### WHAT CAN HAPPEN

Tire may float, causing loss of traction and loss of control.

Which could lead to an accident.

##### HOW TO AVOID THE HAZARD

Never operating the ATV through water which exceeds the recommended maximum depth in this manual.

Avoid operating the ATV through deep or fast flowing water. If you cannot avoid water, go slowly, balance your weight carefully avoiding sudden movements, maintain a slow and steady forward motion, do not make sudden turns or stops, and do not make sudden throttle changes.

Remember that wet brakes may have reduced stopping ability. Test your brakes after leaving water. If necessary, apply them several times to let friction dry out the pads.

## WARNING

##### POTENTIAL HAZARD

Improperly operating in reverse.

WHAT CAN HAPPEN

You could hit an obstacle or person behind you , resulting in severe injury.

HOW TO AVOID THE HAZARD

When you select reverse gear, make sure there are no obstacles or people behind you. When it is safe to proceed, go slowly.

## WARNING

##### POTENTIAL HAZARD

Operation this ATV with improper tires, or with improper or uneven tire pressure.

##### WHAT CAN HAPPEN

Use of improper tires on this ATV, or operation of this ATV with improper or uneven tires pressure, may cause loss of control, and increase the risk of an accident.

##### HOW TO AVOID THE HAZARD

Always use the size and type tires specified in the owner’s manual for this vehicle.

Always maintain proper tire pressure as described in the Owner’s Manual.

## WARNING

POTENTIAL HAZARD

Operation this ATV with improper modifications.

WHAT CAN HAPPEN

Improper installation of accessories or modification of this vehicle may cause

changes in handling which in some situations could lead to an accident.

HOW TO AVOID THE HAZARD

Never modify this ATV through improper installation of use of accessories. All parts and accessories added to this vehicle should be genuine or equivalent components designed for use on this ATV; and should be installed and used according to instructions. If you have question, consult an authorized dealer

# WARNING

POTENTIAL HAZARD

Overloading this ATV or carrying or towing cargo improperly.

WHAT CAN HAPPEN

Could cause changes in vehicle handing, which could lead to an accident.

HOW TO AVOID THE HAZARD

Never exceed the stated load capacity for this ATV.

Cargo should be properly distributed and securely attached.

Reduce speed when carrying cargo or pulling a trailer.

Allow greater distance for braking.

Always follow the instruction in the owner’s Manual for carrying cargo or pulling a trailer.

# WARNING

##### POTENTIAL HAZARD

Riding on frozen lakes and rivers.

##### WHAT CAN HAPPEN

Severe injury or death can result if the ATV and/or the operator brake through the ice.

##### HOW TO AVOID THE HAZARD

Never ride your ATV on a frozen body of water before you are sure the ice is thick enough and sound enough to support the machine and its operator, as well as the force that is created by moving vehicle.

After a rollover or an accident, have a qualified service dealer check the complete machine including, but not limited to, brakes, throttle and steering for possible damage.

# WARNING

Safe operation of this rider active vehicle requires good judgment and physical skills. Persons with cognitive or physical disabilities who operate this vehicle have an increase risk of overturns and loss of control, which could result in severe injury or death.

## WARNING

WARNING

Keep combustible materials away from exhaust system.

Fire many result.

6.V.I.N.

Record these number from your ATV in the spaces provided.

- DETAIL A: Frame VIN (right middle side of engine crankcase)

- DETAIL B: Nameplate (found in the right back side of the frame tube)

- DETAIL C: Engine serial number (found in the right middle side of the frame tube)

Remove the spare key and store on a safe place.

Your key can be duplicated only by obtaining a key blank and having it cut

By mating it with your existing key.

Record key number

The vehicle frame and engine serial number are important for model identification when registering your vehicle, obtaining insurance or whenever replacement parts are enquired. In the event your vehicle were stolen these numbers are essential to the recovery and identification of your ATV

7.CONTROL AND PARTS FUNCTIONS

Electrical switces

- Main Switch-This key switch must be turned clockwise to the “on” position to start the engine.

- Emergency Engine Stop Switch-the engine will not start or run when the switch is its “off” position. Its purpose is to provide the operator with a quick means of engine shutdown in case of stuck throttle or other emergency. NOTE: to stop the engine, slide the stop switch to the “off” position.

3. Engine start button- to start the engine. (at the left back of the handle bar)

4. High beam light switch

5. Turn light switch

6. Horn switch

##### Light switches and indicator lights

## WARNING

This ATV is not equipped highway approved lighting.

This ATV is designed for off- road use only and must

Not be ridden on streets or highways. Use caution and

Drive at reduced speeds in conditions of reduced visibility

Such as fog, rain and darkness.

Indicator Lights

This ATV has indicator lights. The configuration of these lights

Differs with individual models and not every model is equipped with all

The lights. The information will help you identify the lights on your Machine and their function.

NOTE: The taillight is on whenever the main switch (key) is in

The “on” position. Turn the key to prevent battery drain.

LCD

##### The position of the LCD

The LCD is at the head of the ATV,between two handles.

##### The  function of the LCD

1.To display the speed of ATV.

2.To display total distance you have rode.

3.To display the distance you have road in one time.

4.To display the distance you have road this time.

5.To display the total time you have rode your ATV.

6.To display the time you have rode this time.

7.To display the clock

8.To display the capacity of fuel in your fuel tank

9.To display the warning if the temperature of water is too high

10. To display the mode you ride now

11. To display the transmission gear you select now(L/H/N/R/1/2/3/4/5)

12. display Light：High beam light、fan indicator light、left/right turning light

Throttle.

## WARNING

Do not start or operate an ATV with sticking or improperly operating throttle controls. A stuck or improperly operating throttle could cause an accident resulting in severe injury or death. Always contact your dealer for service repairs whenever throttle problem arise.

Failure to check or maintain proper operation of the throttle system can result in the throttle lever sticking during ridding and cause an accident. Always check the lever for free movement and return before

Staring the engine and occasionally during riding.

Throttle lever

Engine speed and vehicle movement

are controlled by pressing

the throttle lever. The throttle lever(1)

is spring loaded and engine speed

returns to idle when the lever is released.

## WARNING

Washing or operating the ATV in freezing temperatures can result in water freezing in the throttle cable conduit and/or on the throttle mechanism.

This may result in the throttle sticking which can cause the engine to continue to run result in loss of control.

Front and rear brakes

The brake fluid in the master cylinder, which is located on the left handlebar, should be checked before each ride. There is an indicator window(1) on the top of the master cylinder. This window will appear dark when the fluid lever is full. When fluid needs to be added, the window will be clear.                                                            “top window”

NOTE: when checking the fluid level, the ATV must be on lever ground and the handlebars must be straight. If the fluid lever is low add DOT3 only.

On some modes, there is a “side window”, the fluid lever can be seen through it, and should be maintained between the indicated “ max” and “min” marks on the reservoir.

## WARNING

Once a bottle of brake fluid is opened, use what is necessary and discard the rest. Do not store or use a partial bottle of brake fluid. Brake fluid is hygroscopic; meaning it rapidly absorbs moisture from the air. This causes the boiling temperature of the brake fluid to drop, which can lead to early brake fade and the possibility of severs injury.

Front and rear brakes

The front and rear brakes are applied by squeezing the left side brake lever qoward the handlebar. The front and rear brakes are hydraulically activated disc type brakes which are activated by one lever only.

Always test brake lever travel and reservoir fluid lever before ridding when squeezed, the lever should feel firm. Any sponginess would indicate a possible fluid leak or low master cylinder fluid lever, which must be corrected before riding. Contact your dealer for proper diagnosis and repairs.

WARNING

Never operate the ATV with a spongy feeling brake lever. Operating the ATV with a spongy brake lever can result in loss of braking. Loss of braking could cause an accident.

##### Setting the Parking Brake

●Machine Packing Brake

1.  Squeeze the hand brake, pull the shaft on the packing brake from level 1 to level 2 just like the picture shows.

2.  Release the hand brake, the kart is packed.

3.  When you want to start the engine and drive the kart, please push the shaft back to level 1.

●Hydraulic Pressure Packing Brake

- Squeeze the left hand brake lever two or three times and hold it.

- Push the park brake lock (1) into the notches on the master cylinder body release the brake lever.

- To release the parking brake lock, squeeze the brake lever. It will return to its released position.

##### Important safeguards

  - The parking brake may relax when left on for a long period of time. This could cause an accident.

  - Do not leave the vehicle on hill depending on the parking brake for more than five minutes.

  - Always block the downhill side of the wheels if leaving the ATV ON A HILL OR PARK THE ATV in a side hill position.

WARNING

Always check to be sure that parking brake has been disengaged before operating the ATV. An accident could result causing severe injury if the parking brake is left on while the ATV is operated.

Auxiliary brake

WARNING

Use caution when applying the auxiliary brake. Do not aggressively apply the auxiliary brake when going forward or the rear wheels may skid and slid and slid sideways causing loss of control.

You ATV has an auxiliary brake provided as a safety feature. It is located on the inside of the right floor board and is operated by the right foot. It is intended as a backup to the main brake system, especially if the main system becomes inoperative. If the rear wheels slide, rescue brake pedal pressure to brake the rear wheels without skidding. Aggressively applying the rear brake when backing down a hill may cause rear tip over.

##### Brake fluid lever

It will be necessary to check the brake fluid lever for the auxiliary brake system. The reservoir is located near the foot brake. The fluid should be kept between the maximum and minimum marks.

Automatic transmission gear selector operation

The transmission gear selector is located on the right side of the vehicle, the transmission selector lever has four positions: forward high speed; forward low speed ,reverse; and neutral.

WARNING

To change gear, stop the vehicle and with the engine idling, move the lever to the desired gear. Shifting gears with the engine speed above idle or while the vehicle is moving could cause transmission damage.

Always place the transmission in gear with the parking brake locked whenever the vehicle is left unattended.

Maintaining shift linkage adjustment is important to assure proper transmission function. Should you experience any shifting problem see your dealer.

## WARNING

POTENTIAL HAZARD

Engaging a lower gear when the engine speed is too high.

WHAT CAN HAPPEN

The wheels could stop rotating. This could loss of control, an accident and injury. It could also cause engine or drive train damage.

HOW TO AVOID THE HAZARD

Make certain the engine has sufficiently slowed before shifting to a lower gear.

Engine cooling system coolant level

The recovery bottle, located under the seat, must

be maintained between levels indicated on the recovery bottle.

The engine coolant lever is controlled or maintained by the recovery system. The recovery system components are the recovery bottle, radiator filler neck, radiator pressure cap and connecting hose.

As coolant-operating temperature increases, the expanding (heat-ed) excess coolant is forced out of the radiator past the pressure cap and into the recovery bottle. As engine coolant is drawn back up from the thank past the pressure cap and into the radiator.

NOTE: some coolant level drop on new machines is normal as the system is purging itself of trapped air. Observe coolant levels and maintain as recommended by adding coolant to the recovery bottle. We recommended the use of a 50/50 mixture of high quality aluminum compatible anti-freeze coolant and distilled water

NOTE: always follow the manufacturer’s mixing recommendations for the freeze protection required in your area.

Cooling system

## WARNING

Never remove the pressure cap when the engine is warm or hot. Escaping steam can severe burns. The engine must be cool before removing the pressure cap.

Radiator coolant level inspection

NOTE: this procedure is only required if the cooling system has been drained for maintenance and/or repair. However, if the recovery bottle has run dry, the level in the radiator should be inspected and coolant added if necessary.

NOTE: use of a non-standard pressure cap will not allow the recovery system to function properly. If the cap should need replacement contact your dealer for the correct replacement part, to insure that the coolant maintains its ability to protect the engine, it is recommended that the system be completely drained using a funnel, slowly add coolant as necessary through the radiator filler neck.

##### Fuel and oil system

## WARNING

Gasoline is highly flammable and explosive under certain conditions.

- Always exercise extreme caution when ever handling gasoline.

- Always refuel with the engine stopped and outdoors or in a well-ventilated area.

- Do not over fill the tank. Do not fill the tank neck.

- Do not smoke or allow open flames or sparks in or near the

- Area where refueling is performed or where gasoline is stored.

- If you get gasoline on your skin or clothing, immediately wash it off with soap and water and change clothing.

- Never start the engine or let it run in an enclosed area. Gasoline powered engine exhaust fumes are poisonous and can cause loss of consciousness and death in a short time.

- Shut off fuel valve whenever the ATV is stored or parked.

## WARNING

The engine exhaust from this product contains chemicals known, in certain quantities, to cause cancer, birth defects or other reproductive harm.

##### Fuel system

The fuel tank filler cap (1) is located directly behind the

handlebar. Refer to your owner’s manual for tank capacity.

Use regular unleaded gasoline.

The fuel valve is located on the left side of the front fender and

has three positions.

OFF: for vehicle storage and whenever transporting

On: for normal operation.

Res: for reserve supply in the event of main supply exhaustion.

Note: here is about a 6 mile (10km) range on reserve gas. Always refill the gas tank as soon as possible after having used reserve supply.

Always return valve to” on” position after refueling machine.

Fuel filter

This filter should be replaced by your dealer every 100 hours of operation or annually. Do not attempt to clean the fuel filter.

Oil System

The oil tank is located on the right side of the engine.

To check the oil:

- Set machine on a level surface.

- Start the engine and let it idle for 20-30 seconds.

- Stop the engine, remove dipstick(1) and wipe dry with a clean cloth.

- Put dipstick into the oil tan k (don’t screw in it), remove it and read the oil level.

- Remove dipstick and check to see that the oil level is between the full and add marks (2) add oil as indicated by the level on the dipstick. Do not overfill.

## WARNING

Use only SAE 15 w/40,SG Oil. Never substitute or mix oil brands. Serious engine damage and voiding of warranty can result.

8. STARTING THE ENGINE

##### Procedure For Starting a Cold Engine

## WARNING

Never run an engine in an enclosed area. Carbon monoxide exhaust gas is poisonous ans can cause severe injury or death. Always start engine outdoors.

CAUTION

You must allow your vehicle adequate warm up time before operating or engine damage could result.

- Place the transmission in neutral and reset the parking brake.

- Turn the fuel tank valve to ON.

- Sit on the vehicle.

- Turn the engine stop switch to RUN.

- Turn the ignition key to ON, apply the brake lever and press the starter button.

- Do not press the throttle more than 20% while starting the engine.

- Activate the starter for a maximum of five seconds, releasing the button when vehicle starts. If engine does not start, release the starter for anther five seconds. Repeat this procedure until engine starts.

CAUTION

This ATV only equipped with an electric start system.

If the battery is under charging, the ATV will not run.

9.VEHCLE BRAEAK-IN PERLOD

The break-in period for your new is defined as the first 50 hours of operation. No single action on your part is as important as a proper break in period. Careful treatment of new engine will result in more efficient performance and longer life for the engine. Perform the following procedures carefully.

CAUTION

Do not operate at full throttle or

high speeds for extended

periods during the break-in

period. Excessive heat can build

up and cause damage to close

fitted engine parts.

Please set a limit to half throttle during the break- in periods.

1.Locknut    2. Adjuster

- Fill fuel tank.

- Check oil reservoir level indicated on dipstick. Add oil if necessary.

- drive slowly at first, select an area which is open and will give you room to familarize yourself with vehicle operation and handling

- Vary the throttle positions. Do not operate at sustained idle

- perform regular checks on fluid levels, controls and all important areas on the vehicle as outlined earlier I=on the daily pre-ride inspection checklist found in”4.daily pre-ride inspection”.

- Don’t pull loads.

- Break in oil and filter. Change at 20 hours or 500 miles/800km

10. RIDING WEAR

Safe riding wear

Always wear clothing suited to the type of riding you are doing. ATV riding requires special protective clothing that will make you feel more comfortable and reduce chances of injury

1.helmet

Your helmet is the most important piece of protective gear for safe riding. A helmet can prevent a severe head injury.

Select an approved helmet.

2. Eye protection

A pair of goggles or helmet face shield offers the best protection for your eyes.

3. Gloves (off-road style)

4. Boots

A pairs of strong over the calf type boots with heels, such as moto-cross boots

5.Clothing

To protect your body, long sleeves and pants should always be worn. Riding pants with kneepads, a jersey and shoulder pads provide the best protection.

11.CARRYING LOADS

Your ATV has been designed to carry a certain amount of load. CARGO WEIGHT should de evenly distributed (1/3 on the front and 2/3 on the rear) and mounted as possible. When operating over rough or hilly terrain, reduce speed and cargo to maintain stable driving conditions. Never exceed the weights specified in your Owner’s manual.

Maximum trailer weight-450 Ibs (200kg) on level ground. Maximum vertical hitch weight-25 Ibs(11kg)

Improper loading of the front rack can obstruct the headlight beam, reducing night visibility. Do not obstruct the headlight beam with cargo.

WARNING

orrect loading of this vehicle is necessary to maintain proper stability and operating characteristics. Overloading or incorrect positioning of the load affects the vehicle’s turning. Stopping distance and stability. Failure to follow loading requirements could cause severe injury or death.

Important safeguards

To reduce risk of injury or machine damage when carrying loads. Read and follow the warning listed below.

- Reduce speed and allow greater distance for braking when carrying cargo.

- Cargo weight distribution should 1/3 on the front rack and 2/3 on the rear rack. When operating over rough or hilly terrain, reduce speed and cargo to maintain stable driving conditions. Carrying loads on one rack only increases the possibility of vehicle tip over.

- Heavy loads can cause braking and control problems. Use extreme caution when applying brakes with a loaded vehicle. Avoid terrain or situation whish may require backing downhill.

- All loads must be secured before moving vehicle. Unsecured loads can create unstable operating conditions which could result in loss of control of the vehicle.

- Loads must be carried as low on the racks as possible. Carrying loads high on the racks raises the center of gravity of the vehicle and creates a less stable operating condition. When cargo loads are carried high on the racks, the weight of the loads must be reduced to maintain stable operating conditions.

- Operate only weigh stable and safely arranged loads. Avoid handling off-centered loads which cannot be centered. Always attach the tow load to the tow load to the hitch opint designated for your ATV.

- Extreme caution must be used. Avoid operating with loads extending over the rack sides. Stability and maneuverability may be adversely affected, causing the vehicle to overturn

- Do not block the headlight/taillight and the reflectors when carrying loads on the racks.

- Do not travel faster than the recommended speeds. Vehicle should never exceed 10 mph (16km/h) while towing a load on a level grass surface. Vehicle speed should never exceed 5 mph(8km/h) when towing loads in rough terrain, while cornering or while ascending or descending a hill.

12.RIDING

WARNING

You must inspect your ATV each time before riding to ensure it is in proper working order, if proper inspection is not done, severe injury or death could result.

See “4.DAILY PRE-RIDE INSPECTION”

- Sit upright with both feet on the footrests and both hands on the handlebars.

- After starting the engine and allowing it to warm up, shift the transmission into gear.

- Check your surroundings and determine your path of travel.

- Release the parking brake.

- Slowly depress the throttle with your right thumb and begin driving. Vehicle speed is controlled by the amount of throttle opening.

Drive slowly, practice maneuvering and using the throttle and brakes on level surfaces.

##### Making turns

Practice making turns at slow speeds.

This ATV is equipped with a solid rear axle which drives both rear wheels equally at all times. This means that the wheel on the outside of the turn must travel a greater distance than wheel on the outside of the turn must travel a greater distance than the inside wheel when turning and the inside tire must slip traction slightly. To turn, steer in the direction of the turn leaning your upper body to the inside of the turn while supporting your weight in the outer footrest. This technique alters the balance of traction between the rear wheels allowing the turn be made smoothly.

The same leaning technique should be used for turning in reverse.

WARNING

Avoid turning at sharp angles in reverse as tip over and severe injury may result.

##### Riding on slippery surfaces

Whenever riding on slippery surfaces such as wet trails or loose gravel, or during cold freezing weather, special attention must be paid to prevent vehicle turnover.

Always:

- Slow down when entering slippery areas.

- Maintain a high level of alertness, reading the trail and avoiding quick, sharp turns which can cause skids.

- Correct a skid by turning the handlebars in the direction of the skid and shifting your body weight forward.

- Never apply brakes during a skid, complete loss of ATV control can result.

- Do not operate on excessively slippery surfaces.

- Always reduce speed and use additional caution.

WARNING

Failure to exercise care when operating the ATV on slippery surfaces can be dangerous.

Loss of tire traction and vehicle control can result in an accident, including an overturn.

Traveling uphill

WARNING

Exercise extreme caution when traveling in hilly terrain. Braking and handling are greatly affected. Loss of vehicle control of overturning of the ATV could occur causing severe injury or death.

Whenever traveling uphill always travel straight uphill and:

- Avoid steep hills (15% maximum).

- Keep both feet on the footrests.

- Transfer your weight forward.

- Proceed at a steady rate of speed and throttle opening.

- Remain alert and be prepared to take emergency action.

- This may include quick dismounting of the ATV.

Side hilling

Side hilling your ATV is one of the most dangerous types of riding and should be avoided. If you do enter into a situation where side hilling is necessary, always:

- Slow down.

- Lean into the hill transferring your upper body weight toward the hill while keeping your feet in the footrests.

- steer slightly into the hill to maintain vehicle directions.. if vehicle begins to tip, quickly turn the front wheel downhill. If possible, or dismount on the uphill side immediately!

WARNING

Improperly crossing hills or turning on hills can be dangerous loss of vehicle control or overturning of the ATV could occur causing severe injury or death.

##### Traveling downhill

- Drive directly downhill.

- Transfer your weight to the rear of the vehicle.

- Slow down.

- Apply the brakes slightly to aid in slowing. Familiarize yourself with the auxiliary rear brake pedal and its use in the event loss of normal service brakes occurs.

WARNING

Do not travel at excessive speeds. It is dangerous and can cause loss of vehicle control and tipping, resulting in severe injury or death.

Turning Around On A Hill

If the ATV stalls while climbing a hill, never back it down the hill. One maneuver that can be used when it is necessary to turn around while climbing a hill is the k-turn.

- Stop and lock the parking brake while maintaining body weight uphill.

- Leave transmission in forward and shut off engine.

- Dismount on left or uphill side of ATV.

- Staying uphill of ATV, turn handlebars full left (while facing front of ATV).

- While holding service brake, release parking brake lock and slowly allow ATV to roll around to your until ATV is pointing across the hill or slightly downward.

- Lock the parking brake and remount ATV from the uphill side, maintaining body weight uphill.

- Restart engine with transmission still in forward, release-parking brake, and proceed slowly, controlling speed with the service brake, until ATV is on reasonably level ground.

WARNING

Avoid climbing steep hills, loss of vehicle control or overturning of the ATV could occur resulting in severe injury or death.

Crossing Streams

Your ATV can operate through water up to maximum recommended depths is 8inch. Before fording streams always:

- Determine water depths and current.

- Choose a crossing where both banks have gradual inclines.

- Proceed slowly, avoiding rocks and obstacles if possible.

- After crossing, dry the brakes by applying light pressure to the lever until braking action is normal.

WARNING

Never operating the ATV through deep or fast flowing water.

NOTE: After running the vehicle in water, it is critical your machine is serviced as outlined in the maintenance chart see”16. Maintenance”. The following areas need special attention: engine oil, transmission oil. Rear gear cased, and all grease fittings.

WARNING

If your ATV becomes immersed, take it to your sealer before starting the engine. Major engine damage can result if the machine is not thought inspected.

If it is impossible to take it to a dealer before starting. Follow the steps outlined below.

- Move the ATV to dry land or at the very least, to water depths not more than 8 inch (200mm).

- Turn the fuel valve to “OFF”.

- Remove the spark plug.

- Loosen the carburetor drain screw(1)

- Turn the engine over several tomes

- with electric start.

- Dry the spark plug and reinstall or replace

- with a new plug.

- Tighten the carburetor drain screw (1)

- Turn the fuel valve to “ON”

- Attempt to start the engine. If necessary

- Repeat the “drying” procedure.

- Take the machine to your dealer for service as soon as possible whether you succeed in stating it or not.

If water has been ingested into the CTV system, take the ATV to your dealer for service as soon as possible.

##### Trail obstacles

Keep Alert!

Look ahead and learn to read the trail as you ride. Stay on the right side of the trail, if possible, and be constantly alert for hazard such as logs, rocks and low hanging braches.

WARNING

Not all obstacles are visible. Travel with caution on trails. Severe injury or death can result if your vehicle comes in contact with a hidden obstacle.

WARNING

Backing your ATV can be dangerous!

You could hit an obstacle or person behind you; or the vehicle could tip over rearward on a steep incline causing severe injury or death.

##### Backing up

- Avoid backing up on steep inclines.

- Always back slowly

- When in reverse, apply the brakes lightly for stopping.

- Avoid turning at sharp angles in reverse.

WARNING

Opening the throttle more than required may cause excessive fuel to build in the exhaust, resulting in engine popping and/ or engine damage.

Parking on an incline

Whenever the vehicle is parked

- Turn the engine off.

- Place the transmission in gear

- Set the parking brake.

- Shut off fuel supply.

- Avoid parking on an incline. If it is necessary to park on an incline, always block the rear wheels on the downhill side as shown above.

- Don’t leave the ATV on a hill depending on the parking brake for more than five minutes.

13. CTV SYSTEM

##### CTV System

WARNING

The CTV system rotates at high speeds, creating large amounts of force on clutch components. However, as the owner, you have the following responsibilities to make sure this system remains safe:

- Do not modify any component of the CTV system. Doing so may reduce its strength so that a failure may occur at high speeds. Any modification will cause the system to be out of balance, creating vibration and additional loads on components.

- Routine maintenance is the responsibility of the owner. Always follow recommended maintenance procedures. See your dealer.\

- The CTV housing must be warning can result in severe injury or death.

Low range use may reduce CVT operating temperatures

The basic operation of the CTV system is dependent on engine speed and vehicle torque requirements. As engine speed increased. The force exerted on the movable drive sheave by the fly- weights also increases. This in turn increases the amount of “ pinch” applied to the drove belt. Similarly, if the engine speed decreases, the amount of centrifugal force decreases, educing the amount of belt” pinch”.

14. BATTERY

##### Battery

Whenever removing the battery , disconnect the negative (black) cable first . when reinstalling the battery , connect the negative (black) cable last or an explosive situation could result causing serious injury or death.

WARNING

Battery electrolyte is poisonous. It contains sulfuric acid. Serious burns can result from contact with skin, eyes or clothing. Antidote:

External: Flush with water.

Internal: Drink large quantities of water or milk. Follow with milk of magnesia, beaten egg, or vegetable oil. Call physician immediately. Eyes flush with water for 15 minutes and get prompt medcial attention .Batteries produce explosive gases .Keep saprks,flames ,cigarettes ,etc.away .

Ventilate when charging or using in an enclosed space.Always shield eyes when working near batteries.

KEEP OUT OF TEACH OF CHILDREN.

##### Replenishing the battery fluid

A pooring maintained battery will deteriorate rapidly ,Check the battery fluid level often .The fluid level should be kept between the upper and the level marks.

To refill use only distilled water.

Tap water contains minerals which are harmful to a battery.

##### Battery removal

WARNING

!!!!!!

1. Disconnect hold down straps holding the electrical box and battery in position and remove battery cover.

2.Remove the battery vent tube from the battery.

3.Disconnect the black (negative)battery cable first.

4.Disconnect the red( positive )battery cable next.

- Lift the battery out of the ATV ,being careful not to tip it sideways and spill electrolyte.

WARNING

If electroyte spills ,immediately wash it off with a solution of one tablespoon baking soda and one cup water to prevent damage to the ATV.

Battery installation and connections

WARNING

To avoid the possibility of the explosion ,always connect battery cables in the order specified.Red (positive)cable first;black (negative)cable last.An exploding battery can cause serious injury or death.

Battery terminals and connections should be kept free of corrosion ,if cleaning is necessary,remove the corrosion with a stiff wire Brush.Wash with a solution of one tablespoon baking soda and one cup water .Rinse well with tap water and dry off clean rags .coat the terminals with grease or petroleum jerry.Be careful not to allow cleaning solution or tap water into the battery.

14．BATTERY

1.Set the battery in its holder .

2.Install the battery vent tube,It must be free from obstructions and securely installed.If not ,battery gases could accumulate and cause an explosion ,The tube should be routed away from the frame and body to ptevent corrosion .Avoid skin contact with electrolyte ,severe burns could result .

3.First connect and tighten the red (positive)cable.

4.Second connect and tightern the black (negative)cable.

5.Reinstall battery cover and attach the hold down strap.

6.Verify that cables are properly routed .

NOTE:

● When your ATV is palced in storage for one months or more,the battery should be removed ,charged to proper level ,and stored in a cool dry place,

●  Before reusing ,take the battery to your dealer for testing and recharging .

●  Power plug leads may need to be bent down so that the battery cover may be installed .

●  When installing a new battery ,make certain it is fully charged prior to it’s initial use,using a new battery that has not been fully charged can damage the battery resulting in a shorter life of the batterey.It can also hinder vehicle performance.

WARNING

Your ATV is equipped with a 14AH battery ,this may no be sufficient to provide power for optional

Equipment.when installing optional equipment ,please upgrade your battery as necessary .See your

Dealer for the proper Battery.

15.EXHAUST SYSTEM

SYSTEM REGULATION

TAMPERING WITH NOISE CONTRO SYSTEM PROHIBITED!

CAUTION:Exhaust system components are very hot during and after use of ATV.

●  Do not touch exhaust system components ,Serious burns can be result .Be especially carefu when traveling through tall grass .The potential for fire exiets.

Spark Arrestor

The exhaust pipe must be periodically purged of accumulated carbon as follows:

1.Remove the arresor screw ① located on the bottom of the muffle,pull out the arrestor (the  mesh)②

2.Clean the arrestor or replace it.

WARNING

When cleaning the spark arrestor ,you must follow the safe guards lisited below to avoid serious injury.

Do not perform this operation immediately after the engine has been run because the exhaust the system be comes very hot .

Keep combustible materials away from exhaust system.Fire may result.

16.MAINTENANCE

WARNING

DD

Due to the nature of the adjustments marked with   a on the following chart ,it is recommendend that service be performed by an authorized dealer.

More often under use ,such as dirty or wet conditions .To purge water or dirt contamination from grease fittings and other critical components.

##### Periodic Maintenance schedule

Careful periodic maintenance will help keep your vehicle in the safest ,mist reliable condition.Inspection ,adjustment and lubrication intervals of important components are explained in the following chart on the following pages .

Maintenance intervals are based upon average riding conditions and an average vehicle speed of appromately 10 miles per hour .Vehicles subjected to severe use ,such as operation in wet or dusty areas ,should be inspected and serviced more frequently.

Inspect ,clean ,lubricate ,adjust or replace parts as necessary .

NOTE:Inspection may reveal the need for replacement parts.Always use genuine parts available from your dealer.

Service and adjustments are critical ,If you are not familiar with safe service and adjustments procedures ,have a qualified dealer perform these operations.

WARNING

|  | Item | Hours | When | Remarks |
| --- | --- | --- | --- | --- |
|  | Brake system | Per-ride | Per-ride | Pre-ride inspection item |
|  | Auxillary brake | Pre-ride | Per-ride | Pre-ride inspection item |
|  | Tires | Per-ride | Per-ride | Inspect daily ,per-ride inspection |
|  | Wheels | Pre-ride | Pre-ride | Pre-ride inspection item |
|  | Framesnuts bolts fasteners | Pre-ride | Pre-ride | Pre-ride inspection itekm |
|  | Air ,filter-per-cleaner | Daily | Daily | Inspect-clean |
|  | Coolant/level inspection | Daily | Daily | Replace engine coolant every one year |
|  | Air box sediment Tube | Daily | Dailiy | Drain deposits when-ever visible |
|  | Headlamp inspection | Daily | Daily | Check operation daily apply dielectric,Grease to connector when replaced |
|  | Tail lamp inspection | Daily | Daily | Check operation daily apply dielectrc,Grease to connector when replaced |
|  | Air filiter-main Element | Weekly | Weekly | Inpect –replace if necessary |
|  | Transmission oil level | 20hrs | Monthly | Inspect monrth ,change annually |
|  | Battery | 20hrs | Monthly | Check/clean termianls check fluid level |
| D | Brake pad wear | 10hrs | Monthly | Inspect periodically |
|  | Rear gear case oil | 100hrs | Monthly | Check monthly and change annually |
|  | Engine cylinder head and clinder base fasters | 25hrs | 3 months | Inspect(re-torque required at first service only) |
|  | General lubrication | 50hrs | 3 months | Lubricate all fittings pivots cables etc |

16.MAINTENANCE

|  | Item | Hours | When | Remarks |
| --- | --- | --- | --- | --- |
|  | Engine oil –level.change | 30hrs | 3 months | Check level daily ,break in service at 1 month ,change oil mire often in cold weather use |
|  | Oil filter | 50hrs | 6 months | Inpect-clean |
|  | Engine breather hose | 100hrs | 6 months | Inspect |
|  | Carburetor float bowl | 50hrs | 6 months | Drain bowl periodically and prior to storage |
|  | Throttle cable | 50hrs | 6 months | Inspect-adjust lubricate,replce if necessary ,pre-ride inspection item |
|  | Coolant strength | 100hrs | 6 months | Inspect strength seasonally |
|  | Shift linkage | 50hrs | 6 months | Inspect ,adjust |
|  | Drive belt | 50hrs | 6 months | Inspect ,replace if necessary |
|  | Steering | 50hrs | 6 months | Inspect daily lubricate |
|  | Rear axle | 50hrs | 6 months | Inspect bearing Lube |
|  | Front suspension | 50hrs | 6 months | Inspect-lubricate tighten fasteners |
|  | Rear suspension | 50hrs | 6 months | Inspect-tighten fasteners |
|  | Spark plug | 100hrs | 12 months | Inspect replace if necessary |
|  | Ignition timing | 100hrs | 12months | Inspect and adjust as needed |
|  | Fuel system | 100hrs | 12 months | Check for leaks at tank cap,lines fuel valve ,filter ,and caburetor ,Replace lines every one year |
|  | Fuel filter | 100hrs | 12 months | Replace annually |

16.MAINTENANCE

|  | Item | Hours | When | Remove |
| --- | --- | --- | --- | --- |
|  | Radiator | 100hrs | 12months | Inspect./clean external surface |
|  | Cooling system | 50hrs | 6 months | Inspect /replace if necessary |
|  | Spark arrestor | 10hrs | Months | Clean out replace if necessary |
|  | Clutches (drive and driven) | 25hrs | 3months | Inspect clean |
|  | Engine | 25 hrs | 3 months | Inspect |
|  | Valve clearance | 100 hrs | 12 months | Inspect/adjust |
|  | Shift selector | 200hrs | 24 months | Change grease every two years |
|  | Brake fluid | 200hrs | 24 months | Change every two years |
|  | Idle speed | Asrequired | As required | Adjust |
|  | Toe adjustment | As required | As required | Periodic inspection adjust when parts are replaced |
|  | Heading Aim | As required | As required | Adjust if necessary |

16,MAINTENANCE

|  | Item | Lude rec | Method | Frequency |
| --- | --- | --- | --- | --- |
|  | 1.engine | SAE 15w/40SG | Add to proper level on dipstick | Check level daily |
|  | 2.brake fluid | DOT3 only | Maintain level between fill line s,See “7.CONTROL…” | As required;change every two years or 200hours |
|  | 3.Transmission | SAE 80W/90GL5 | See:16.MAINTENANCE/TRANSMISSION LUB…) | Charge annually or at 100hours |
|  | 4.rear gear case oil | SAE80W/90GL5 | See”16.MAINTENANCE/rear gear case lubrication…” | Charge annually or at 100hours |

Lubrication recommeddations

|  | 5.Front A-Arm prvot shaft | Grease | Locate fitting in pivot shaft and grease with grease gun | Every 3 months or 50 monthsor 50 hours |
| --- | --- | --- | --- | --- |
|  | 6.Steering post bushings | Grease | Locate fitting on steering rest and grease with grease gun | Every 3 months or 50 hours |
|  | 7.Front wheel bearing | Grease | Inspect and replace bearing if necessary | Semi-annually |
|  | 8.Tie rods | Grease | Locate fitting and graese | Semi-annually |
|  | 9.Shift linkages | Grease | Locate fitting and grease | Semi-annually |
|  | 10.Ball joints | Grease | Inspect and replace it fi necessary | Semi-annually |
|  | 11Universal coupling | Grease | Locate fitting and grease | Semi-annually |
|  | 12.Rear Axles Bearings | Grease | Locate fitting and grease | Every 3 months or 50hours |
|  | 13.thorttle cable | Grease | Grease ,inpect and replace it if necessary | Monthly or 20 hours |

NOTE:

1.More often under severe use ,such as wet or dusty conditons.

M

2.Grease:Light weight lithium-soap grease.

3.Grease :       molybdenum disulfide(Mos2)grease(water resistant).

4.* When susuoension action becomes stiff or after washing.

5.Hours are based on mph(16km/h)average.

##### Periodic Maintenance record

Use the following chart ro record periodic maintenance work:

| Maintenance interval performed | Servicing date | Servicing dealer or person | Remarks |
| --- | --- | --- | --- |
| First 5 hrs |  |  |  |
| 10hrs |  |  |  |
| 15hrs |  |  |  |
| 20hrs |  |  |  |
| 25hrs |  |  |  |
| 50hrs |  |  |  |
| 75hrs |  |  |  |
| 100hrs |  |  |  |

WARNING

Imporper adjustment of the handlebars or incorrect torque of the adjuster block tightening bolts can cause limited steering or loosening of the handlebars,resulting in loss of control and possible serious

Personal injury or death.

##### Hanldebar adjustment

Your ATV has handlebars which can be adjusted for your personal fit.

1.Remove the handlebar cover.

2.Loosen the four bolts.

3.Adjust handlebar to desired height .Be sure handlebars do not hit gas tank or any other part of machine when turned fully to left or right.other part of machine when turned fully to left or right.

4.Torque handlebar adjuster block bolts to 10-12ft.lbs(14-16nm).

NOTE:Tighten bolts so there is an equal gap at the front and rear of the handlebar block ,Improper gap will result in improper fit of upper pod.

The following items should be checked occasionally for tighten;or if they have been loosened for maintenance service.

Wheel nut torque specifications

| Bolts size | Specification |  |
| --- | --- | --- |
| Front m12x1.25 | 50ft.lbs | 69N.m |
| Rear m12x1.25 | 50ft.lbs | 69N.m |

NOTE:All nuts that have a cotter pin installed must be serviced by an authorized dealer.

Front wheel hub tightening.

Front wheel bearing tightness and spindle nut retention are critical component operations.Secrvice work must ne performed by an authorized dealer.

Front:                                       Rear:

Flange nuts:install with flat side against wheel.     Tapered nuts:install with tapered side against

wheel .

##### Rear spring asjustment

The rear shock absorber spring is adjusted by rotating the adjuster in the direction required to increase or decrease spring tension.

Air filter service

1.Remove seat.

2.Release clips and remove cover .

3.Loosen clamp and remove filter.

4.Remove fabric type pre-filter from main filter,wash pre-filter in soapy water and dry it .

5.Reinstall pre-filter pver main filter,replace main filter as required.

##### Steering inspection

The steering assembly of the machine should be checked periodically for loose nuts and bolts ,If loose nuts and bolts are found ,have your dealer tighten them before riding your  vehicle

##### Camber and caster

The camber and caster are non-adjustable.

WARNING

Do not attempt to adjust tie rod for toe alignment .severe injury and death can result from improper adjustment.

Cantact your dealer ,He/she training and tools to make these adjustment.

##### Toe Alignment check

The recommend toe alignment is 1/8 to1/4(3 to6mm)toe out.

1.Set the handlebars in a straight ahead position,and hold them in this position.

2.Measure A and B,A minus B should be1/16 to1/8(1.5 to3mm).

3.MIf this measurement needs to be adjusted .contact your dealer for service.

##### Front Brake

WARNING

Once a bottle of brake fluid is opened ,use what is necessary and discard the rest .Do not store or use a partial bottle ofd brake fluid ,Brake fluid id hygroscopic,meaning it rapidly absorbs moisture from the air,this cause the boiling temperature of the brake fkuid to drop ,Which can lead to early brake fade and the possibility of serious injury.

The front brakes are hydraulic disc brakes which are activated by the brake lever being moved toward the hanblebar,These brakes are self-adjusting and require on adjustmenyt.

The following checks are commended to keep the brake system in good operating condition,How often they need checking depend upon the type of driving that has been done.

● Keep fluid level in the master cylinder reservoirs as described see :7.control and parts functions”Normal functioning of the diaphragm is to extend into the reservoir as fluid level drops ,if the fluid level is low and the diaphragm is not extended,a leak is indicate and the diaphragm should be replaced.Always fill the reservoir as indicated where the cover is loosened of removed to insure proper diaphyragm operation ,use dot 3 brake filuid.

● Check brake system for fluid leaks

● Check brake pads for wear ,damage and looseness.

● Check security and surface condition of the disc.

##### Rear and auxiliary brake

Rear brake

The rear brake is a hydraulic disc you type brake which is acttvated by the same lever which activates the front brake system ,The rear brake system is self adjusting and requires no maintenance rather than periodic checks  of the pads for wear.

●Pads should be changed when the friction material is worn to 3/64”(I mm).

●Inspect the brake disc and pad wear surface for the excessive wear.

Auxiliary brake systems

Your ATV’s auxiliary brake system is intended to used as a backup for the main brake system .Should the main system fail,the rear brake can be activated by depressing the foot pedal on the inside of the right floorboard ,the hydraulic auxiliary brake system will not require adjustment.

NOTE:since this is a rear brake only ,it will not be as effective as the all wheel system.

##### Carburetor/Engine Idle RPM Adjustment

If the engine idle speed is not satisfactory ,and all other condition are favorable ,the carburetor can be adjusted as follows:

1.Warm up the engine by running the vehicle approximately five minutes

2.Place the transmission in gear with the packing brake applied .

3.Adjust the carburetor idle screw in or out unit the desired idle RPMis reached Turning the screw in (clockwise)will raise RPM.Turning the screw out (clockwise)will raise RPM.Turning the screw out (counter clockwise)will lower RPM.

4.the standard idle RPM is 1500+10%

Throttle cable free play adjustment

Throttle cable free play is adjusted at the handlebar.

1.Slide the bolts off online cable adjuster locknut.

2.Turn adjuster until 1/16”to 1/8”(2 to3 mm)free play is achieved at thumb lever .NOTE:while adjusting free play ,it is important you flip the throttle lever back and forth .

3.Tighten locknut and slide boots over cable adjuster.

Spark plugs                            Inspect

●Insulator①

Abnormal color:Replace normal color is

medium-to-light tan color.

●Electrode②

Wear/damage ;replace .

Clean:

Standard spark plug :                  ●Spark plug (with spark plug cleaner or wire DR&EA(NGK)                                 brush )                              ●Measure:

③Gap:0.6-0.7mm                     ●Spark plug gap③

Outer of specification:Asjust gap

##### Spark plug removal replacement

WARNING

Never attempt to remove a spark plug while the engine is warm .The exhaust  system or engine could burn you causing severe injury .

Remove the spark plug by rotating countercloclwise.

Reverse the procedure for spark plug installation .Torque to 17ft.lbs(23N.m)

##### Oil and filter change

The recommended oil interval is 30hours ,or every 3 months ,whichever comes first .suggested break in oil change is at 20 hours ,of the months,whichever comes first ,Severe use operation requires more frequent service ,Severe use includes continuous duty in dusty or wet ,conditions and cold weather riding .

NOTE: Severe use cold weather riding is all riding below 10°  F*（-12℃），and riding between 10

°F（-12℃）and 30°（0℃）when most trips are slow speed and less than 5 mpt(8km/h)Be sure to change the oil filter whenever changing oil.

WARNING

Oil may be hot ,do not allow hot oil  to come into contact with skin as severe burns may result.

1.Place vehicle on a level surface;

2.Run engine two or three minutes until warm ,stop engine ;

3.Clean area around drain plug.

4.Place a drain pan beneath engine crankcase and remove drain plug;

5.Allow oil to drain completely

- Replace sealing washer o-ring of drain plug.

- Reinstall drain plug and torque to 14ft.lds(19N.m)

##### Transmission Lubrication

Transmission fill plug is located on the right side of the machine .The transmission lubricant level should ve checked monthly or 20hours,whichever comes first .Transmission oil should be changed annually .With the ATV on a level surface ,remove fill plug (1) and check the lubricant level .The correct transmission lubricant to use SAE 80w/90GL5lubricant. And see “16.MAINTENANCE/lubrication recommendations “for the correct fluid level .

Transmission oil changing procedure

1.Remove the four screw on the footwall and gently pull outward for access.

2.Remove fill plug.

3.Remove transmission drain plug located on the bottom left hand side and drain the oil .Catch and

discard used oil properly.

4.Clean and reinstall the drain plug torque to 14ft.lbs ,(20 Nm)

5.Add the correct amount of SAE 8w/90GL5 lubricant until oil reaches the bottom of the filler hole,

6.Check for leaks .

7.Reinstall footwall and screws removed in step 1.

##### Rear gear case ,lubrication

With the ATV on a level surface ,inspect fill plug and visually

inspect the lubricant level.Through the fill hole (1)lubricant

should kept even with the center of the drill point (2)Approximately

15/8”(41mm)below the top of the fill hole.

NOTE:Do not add lubricant to the bottom

of the fill plug threads.

The correct gear case lubricant to use is

SAE8w/90GL5Weight gear lube.

##### Oil changing procedure

1.Remove drain plug (3)Catch and discard used oil properly.

2.Clean and reinstall the drain plug with a new sealing washer and tighten securely (14ft.lbs/20N.m)

3.Remove fill plug and add 10 ounces (300ml)of SAE80W/90GL5 weight gear lubricant and inspect oil level .Oil level should be kept even with the center of the drill point (approxiamtely 15/8’’ 41mm below the top of the fill hole.)

4.Reinstall the fill plug ,tighten securely(14ft.lbs/20N.m)

5.check for leaks

WARNING

Operating your ATV with tires,improperly inflated tires,non-standard tires or improperly installed tires will affect vehicle handing whivh could cause an accident resulting in serious injury or death.

Follow the safeguards listed below to prevent this type of situation.

##### Important safeguards

Maintain proper tire pressure according to charts below,improper tire inflation may affect ATV maneuverability.

Do not use improper tires. The use of non-standard size or type Tires may affect ATV handling.

Make certain the wheels are installed properly. If wheels are improperly installed it could affect vehicle hangding and tire wear.

##### Wheel Removal procedure

  - Stop the engine,place the transmission in gear and lock the parking brake.

  - Loosn the wheel nuts slightly.

  - Elevate the side of the vehicle by placing a suitable stand Under the footrest frame.

  - Remove the wheel nuts and remove the wheel.

| Tire pressure |  |
| --- | --- |
| Front | Rear |
| 25kp/4psl | 25kp/4psl |

Wheel installation

- With the transmission in gear and the parking brake locked, place the wheel in the correct position in the wheel hub, be sure the valve stem is toward the outside and rotation arrows on the tire point toward forward rotation.

- Attach the wheel nuts and finger tighten them.

- Lower the vehicle to the ground.

- Securely tighten the wheel nuts according to the chart found in “16.MAINTENANCE/wheel Nut Torque”

##### Tire inspection

When replacing a tire always use original equipment size and type.

##### Tire Tread Depth

Always replace tires when tread depth is worn to 1/8”(3mm)(1) or less. Please refer to your owner’s Manu for tire specifications.

##### Headlight lamp replacement

WARNING

Keep your headlights and taillights clean. Poor lighting while riding can result in an accident causing severe injury or death.

WARNING

Do not service while headlight is hot. Serious burns may result. Do not touch a halogen lamp with bare fingers. Oil from your skin leaves a residue, causing a hot spot which will shorten the life of the lamp.

##### Lamp Replacement

Headlight lamp replacement

- Use bulb 12v 35w (see figure 1) 35w/35w.

- Turn the screw of the tense flake and remove it (see figure 2).

- Remove the lens together with the reflecting mirror.

- Take off the rubber light basis.

- Chang the bulb.

- Fix the replacing bulb 12v 35w/35w into the bronze light basis of the reflecting mirror,and make sure the bulb’s points fit the light basis’s inside roles.

- Fix the rubber light basis into the bronze light basis of the reflecting mirror, and the 3 points of rubber light basis mast be fixed into the 3 openings of bronze light basis, then turn it so that its channel parallels the length side of reflecting mirror (i.e.surface A/B). Moreover the rubber light basis opening turns left (see fight 3).

- Fix the whole reflecting mirror into lens.

Fix the fasten flake and the screw.

##### Taillight/Brake light Lamp Replacement

If the taillight/brake light does not work

The lamp may need to be replaced.

- Remove the les.

- Remove lamp and replace it with recommended lamp

- Test the taillight/brake light to see that it’s working.

- Reinstall the lens.

##### Indicator lamp replacement

- Remove mounting panel.

- Unplug light from harness,depress locking tabs(A) and remove from pod.

- Install new light and reassemble panel.

##### High beam headlight adjustment

The headlight beam can be adjusted up and down.

- Place the vehicle on a level surface with the headlight approximately 25’(7.6m) from a wall.

- Measure the distance from the floor to the center of the headlight and make a mark in the wall at the same height.

- Start the engine and turn the headlight switch to high beam.

- Observe headlight aim. The most intense part of the headlight beam should be aimed 2”(51mm) below the mark placed in the wall in step 2.NOTE: Rider weight must be included on the seat.

- Loosen pivot bolt and adjust beam to desired position.

Tighten nut and bolt.

##### Cleaning your ATV

Keeping your ATVclean will extend the life of various components.

##### Washing

Never use a high pressure type car wash system, it can damage to the wheel bearing, transmission seals,body panels,brakes and warning labels,and water might enter the engine or exhaust system.

The best and safest way to clean your ATV is with a garden hose and a pail of mild soap and water. Use a professional type-washing mitten,cleaning the upper body first and the lower parts last. Rinse with water frequently and dry with a chamois to prevent water spots. NOTE: If warning labels are damaged, contact your dealer for replacement.

##### Washing

Your ATV can be waxed with any non-abrasive automotive paste wax. Avoid the use of harsh cleaners since they can scratch the Body finish.

WARNING

Certain products, including insect repellants and chemicals, will damage platic surfaces. Care must be taken when using these products near plastic surfaces.

##### Storage Tips

WARNING

Do not start the engin during the storage period. This will disturb the protective film created by fogging.

Cleaning-clean the ATV thoroughly

Fuel-Turn the fuel valve to “OFF” and drain the carburetor bowl completely.

Oil add and filter change-Warm the engine and change oil and filter.

Air filter/air box-Inspect and clean or replace the pre-cleaner and filter. Clean the air box and drain the sediment tube

Inspect all fluid levels-Inspect the following fluid levels and Change if necessary:transmission; brake fluid(change ever two years of as required if fluid looks dark or contaminated)

Fog the engine-Spray a light oil into the cylinder through the speak plug hole.

Check and lubricate cables/Grease-Inspect all cables and lubricate.

Battery maintenance-Remove the battery and add distilled water as required to the proper level. Do not use tap water which may contain minerals that reduce battery life. Apply Dielectric Grease to the terminal bolts and terminals,charge the battery.

Storage area/covers-Set tire pressure and safely support the ATV with the tires 1-2”(25-50mm) off the ground,be sure the storage area is well ventilated. Cover the machine with a ATV cover.

NOTE: Do not use plastic or coated materials. They do not allow enough ventilation to prevent condensation condensation, and may promote corrosion and oxidation.

##### Transporting

Whenever the ATV is to be transport, the following measures should be taken.

- Turn off the engine and remove the key.

- Turn the fuel valve to off.

- Be certain the fuel cap,oil cap,and seat are installed correctly.

- Always tie the frame of the ATV to the transporting unit securely using suitable straps or rope.

- Always place the transmission in gear and lock the parking

17.TROUBLE SHOOTING

NOTE:

The following troubleshooting does not cover all the possible causes of trouble. It should be helpful,however,as a guide to troubleshooting. Refer to the relative procedure in this manual for inspection, adjustment and replacement of parts,adjustment and replacement must be done by your dealer.

##### STARTING FAILURE/HARD STARTING

FUEL SYSTEM

Fuel tank                                     Auto choke

● Empty                                     ●  starter plunger malfunction

● Clogged fuel tank breather hole              ● Wax malfunction

● Deteriorated fuel or fuel containing

Water or foreign material

Carburetor                                  Air cleaner

●  Deteriorated fuel or fuel containing           ● Clogged air cleaner element

●  Clogged pilot jet                           ●  Improper air cleaner setting

●  Clogged air passage                       COMPRESSION SYSTEM

●  Improperly set pilot air screw                Cylinder and cylinder head

●  Clogged pilot air passage                   ●Loose spark plug

●  Improperly sealed valve seat                ●Loose cylinder head

●  Improperly adjusted fuel level                ●Broken cylinder head gasket

●  Clogged starter jet                         ●Broken cylinder gasket

●  Sucked-in air                              ●Worn,damaged or broken cylinder

Valve system                                 Piston and piston ring

●Improperly adjusted valve clearance            ●Worn,piston

●Improperly sealed valve                       ●worn,fatigued or broken piston ring

●Improperly contacted valve and valve seat       ●Seized piston ring

●Improper valve timing                         ●Seized piston ring

●Broken valve spring                           ●Seized or damaged piston

●Seized valve

IGNITION SYSTEM

Battery                                        Ignition system

●Improperly charged battery                            ●Faulty ignition unit

●Faulty battery                                        ●Faulty pick up coil

Fuse                                         ●Broken magneo woodruff key

● Burnt out,improper connection                        ●Switch

Spark plug                                            ●Faulty main switch

●Improper plug gap                                    ●Faulty “ENGIN STOP” Switch

●Worn electrodes                                      Wiring

●Wire between terminals broken                         ● Loose battery terminal

●Improper heat range                                  ● Loose coupler connection

Ignition coil                                           ● Improperly grounded

● Broken or shorted primary/secondary coil               ● Broken wireharness

● Faulty high tension cord

● Broken ignition coil body                        Compression system

●Worn cylinder

Fuel system                                  ●Worn or seized piston ring

●Clogged fuel tank breather hole                       ●Cylinder head gasket broken

●Clogged air cleaner element                          ●Cylinder gasket broken

●Clogged jet                                         ●Carbon deposit build-up

●Improperly adjusted fuel level                         ●Improperly adjusted valve clearance

●Improperly carburetor air vent hose Setting             ●Improperly contacted valve and valve seat

●Faulty valve timing

Ignition system                               Clutch

●Faulty spark plug                                    ● Refer to “CLUTCH

●Faulty high tension cord                          SLIPPING/DRAGGING” section

●Faulty ignition unit                              Valve system

●Faulty pick up coil                                   ●Improper adjusted valve clearance

●Faulty ignition coil

17.TROUBLESHOOTING

OVER HEATING OR OVER-COOLING           OVER- COOLING

OVER HEATING                               Cooling system

Ignition system                               ●Faulty thermost at

●Improperly spark plug gap                            ●Faulty thermo switch

●Improper spark plug heat range                        Brakes

●Faulty ignition unit                                    ●Dragging brake

Fuel system                                   Cooling system

●Improper carburetor setting                              ●Inoperative fan motor

●Improper fuel level adjustment                           ●Faulty thermostat

●Clogged air cleaner element                             ●Faulty thermo switch

Compression system                                    ●Incorrect coolant level (low coolant level)

●Heavy carbon deposit build-up                           ●Faulty radiator(clogged,damage)

●Improperly adjusted valve timing                         ●Faulty radiator cap

●Improperly adjusted valve clearance                      ●Impeller shaft gear malfunction(bent dowel pin,gear)

Engine oil                                              ●Damaged impeller shaft

●Incorrect engine oil level                                ●Disconnected fan motor connector

●Improper engine oil quality (high viscosity)                 POOR SPEED

●Low engine oil quality                                   PERFORMANCE

FAULTY CLUTCH WHEN                  V-belt

ENGINE RUN, ATV DOES NOT RUN           ●Worn

V belt                                      ●Oily v belt

Worn/bent/slipping                           Roller weight

Cam,slider                                   ●Worn/impropel operation Primary/secondary/sheave

●Worn/damaged                                   ●Worn

Compression spring                               CVT SYSTEM

●Damage                                         correct primary clutch position

Gear                                             ● Incorrect engine mount (see 13.ATV system)

●Damage                                   FAULTY BRAKE POOR

CLUTCH SLIPPING                              BRAKING EFFECT

Clutch weight spring                        ●Worn front hub beating

●Worm/loose                                 ●Worn brake pad

Clutch shoe                                      ●Worn brake disc

●Worn/damage                               ●Air in brake fluid leaking brake fluid

●Primary sliding sheave                             ●Faulty master cylinder kit

●Seized                                           ●Loose union bolt

POOR STARTING REFORMANCE               ●Broken brake hose

V-belt                                      ●Oily or greasy brake pad

●Slipping/oily v-belt                            ●Oily or greasy blake disc

Primary sliding sheave                         Compressing spring sheave

●Improper operation                           ●Worn/loose

●Damage                                         Secondary sliding sheave

Clutch shoe                                       ●Improper operation

●Worn/bent                                  ●Worn guide pin grove

●Worn guide pin

18. TOOL

19. SPECIFICATION

| # Capacities |  |  |  |  |
| --- | --- | --- | --- | --- |
| Fuel capacity |  | 14.2L |  | 3.75u.s.g.al |
| Engine oil capacity |  | 1.4L |  | 1.23Quarts |
| Ground clearance |  | 190mm |  | 7.5inch |
| Height |  | 1250mm |  | 49.2inch |
| Length |  | 2100mm |  | 82.7inch |
| Width |  | 1200mm |  | 47.2inch |
| Seat height |  | 865m |  | 34inch |
| Wheelbase |  | 1280mm |  | 50.4inch |
| Turn radius |  | 3277mm |  | 129inch |
| Dry weight |  | 280kg |  | 616lbs |
| Front rack |  | 18kg |  | 40lbs |
| Rear rack |  | 36kg |  | 80lbs |
| Load capacity(combined rider& payload) |  | 150kg |  | 350lbs |
| Tongue weight |  | 11kg |  | 25lbs |
| Tow capacity |  | 200kg |  | 450lbs |
| Drive system |  |  |  |  |
| Drive system | CVT |  |  |  |
| Front tire | AT25*8-12 |  |  |  |
| Rear tire | AT25*10-12 |  |  |  |
| Tire pressure(front) | 25kpa |  | 4psl |  |
| Tire pressure(rear) | 25kpa |  | 4psl |  |
| Brake system |  |  |  |  |
| Front wheel | Double Hydraulic disc brake |  |  |  |
| Rear wheel | Double Hydraulic disc brake |  |  |  |
| Left hand brake | Four wheels |  |  |  |
| Foot brake | Four wheels |  |  |  |
| Parking brake | Rear two wheels (right hand ) |  |  |  |

| Engine |  |
| --- | --- |
| Engine type | CF172MM 4-Stroke,Single Cylinder,SOHC |
| Bore x stroke | 72mm x 60mm |
| Displacement | 244cc |
| Starter system | Electric starter |
| Engine cooling | Both water cool and oil cooled |
| Lubrication | Wet sump |
| Carburetor | CVk30 |
| Ignitions | DC-CDI |
| Spark plug type | DPR7EA-9 |

| Electrical |  |
| --- | --- |
| Battery | 12v 14AH |
| Head light | 35w/35w |
| Brake/tail light | 21w/5w |
| Reverse indicator light | 1w |
| Fan indicator light | 1w |
| High beam light | 1w |
| Neutral position light | 1w |
| Heat alarm light | 1w |

20.WIRING DIAGRAM